A Mini Project report on

BASIC ELECTRONICS

Submitted in partial fulfilment of the requirement for the Degree of

B. Tech.

in

COMPUTER SCIENCE & ENGINEERING

Submitted By

PROTTAY KUMAR ADHIKARY (1912157)

SAURABH SINHA (1912159)

RAJDEEP SINGHA (1913034)

Submitted to

PRABINA PATTANAYAK

ASSISTANT PROFESSOR

DEPARTMENT OF ECE

NIT SILCHAR

# DECLARATION

TOPICS: Voltage Regulator & Clipper and Clamper Circuits using Diodes

We declare that the presented work represents largely our own ideas and work in our own words. Where others ideas or words have been included, we have adequately cited and listed in the reference materials. We have adhered to all principles of academic honesty and integrity.

PROTTAY KUMAR ADHIKARY (1912157)            SAURABH SINHA (1912159)            RAJDEEP SINGHA (1913034)

INDEX

Part I   Voltage Regulator

Part II   Clipper and Clamper Circuits using Diodes

Introduction

Clipper

Types of clipper

How to solve circuits?

Applications of Clippers

Clamper

Types of Clamper

How to solve circuits?

Applications of Clampers

Introduction

A diode is a two-terminal electronic component that conducts current primarily in one direction; it has low (ideally zero) resistance in one direction, and high (ideally infinite) resistance in the other. A semiconductor diode is a crystalline piece of semiconductor material with a p–n junction connected to two electrical terminals, which is mostly used now-a-days.

Diodes are used in many ways. One of them is limiting circuits. They can be combined with resistors to provide simple realizations of limiter function. It can be also used as clamper which are used to shift the input signal above or below the DC reference level without altering the shape of input signal.

In the next section, these two application of diode will be explained.

Clipper

Clipper circuits are the circuits that clip off or remove a portion of an input signal, without causing any distortion to the remaining part of the waveform. These are also known as clippers, clipping circuits, limiters, slicers etc.

Clippers are basically wave shaping circuits that control the shape of an output waveform. It consists of linear and non-linear elements but does not contain energy storing elements.

The basic operation of a diode clipping circuits is such that, in forward biased condition, the diode allows current to pass through it, clamping the voltage. But in reverse biased condition, no any current flows through the diode, and thus voltage remains unaffected across its terminals.

Clipper circuits are basically termed as protection devices. As electronic devices are voltage sensitive and voltage of large amplitude can permanently destroy the device. So, in order to protect the device clipper circuits are used.

Types of clippers

Clippers can be divided broadly into following types:

1. Series Clipper

2. Parallel Clipper

3. Combination or Dual Clipper

1. Series Clipper: In series clipper, the diode is connected in series with the input signal. Series clipper can be further divided into two types:

a. Positive Series Clipper:

Here, the positive portion of the waveform is attenuated.

Fig 2.1: Positive Series Clipper

b. Negative Series Clipper:

Here, the negative portion of the waveform is attenuated.

Fig 2.2: Negative Series Clipper

2. Parallel Clipper: In shunt/parallel clipper, the diode is connected in parallel with the input signal. Shunt clipper can be further divided into two types:

a. Positive Parallel Clipper:

Here, the positive portion of the waveform is attenuated.

Fig 1.3: Positive Shunt Clipper

b. Negative Parallel Clipper:

Here, the negative portion of the waveform is attenuated.

Fig 2.3: Negative Shunt Clipper

3. Dual Clipper: In dual/combination clipper, the diodes are connected in parallel with the input signal, such that both the positive and the negative portion of the waveform are attenuated to the desired level.

Fig 2.4: Dual Clipper

NOTE: The desired level of attenuation is obtained by introducing the factor called Biasing. Here, the cells are added in the circuit, parallel to the input signal for the purpose of biasing. This makes sure that only the unwanted portion of the waveform is clipped off. This further classifies the positive and negative series and parallel clippers into following parts:

- Biased Positive Series Clipper

- Biased Negative Series Clipper

- Biased Positive Parallel Clipper

- Biased Negative Parallel Clipper

- Biased  Dual Clipper

The working of the biasing can be clarified with some of the following solved problems and examples.

How to solve clipper circuits?

As we know, there are two general categories of clippers: series and parallel. The series configuration is defined as one where the diode is in series with the load as shown below, while the parallel variety has the diode in a branch parallel to the load.

Fig 2.4: Dual Clipper

Step 1:

Let us write down the KVL equation for the circuit above,

vi─ vD ─ iDR = 0

or, vD = vi ─  iDR

For iD=0,

VD = Vi

ON;	if vi ≤ VD(ON)

OFF;	if vi < VD(ON)

{

So, we have,

Diode state=

vi─VD(ON),	if vi ≥ VD(ON)

0,		if vi < VD(ON)

{

As the output vo is across the resistor R, i.e., v0 = iDR = vi ─ vD, we have

V0 =

Step 2:

Let us plot the equation above, as a voltage transfer characteristics (VTC) curve, i.e., output versus input plot, in order to understand the clipper behaviour visually, as shown below.

Fig 2.4: Dual Clipper

Step 3:

Let us analyse the operation of the series clipper circuit above for a sinusoidal input, using

the ideal diode model, i.e., VD(ON) = 0.

Fig 2.4: Dual Clipper

As we see from the figures above, negative half-cycle portion of the signal is clipped off

while the positive half-cycle portion remains intact.

An example:

By adding a DC source to the circuit as shown, the voltage required to forward bias the diode can be changed.

vi+5V─VD(ON),	if vi ≥ VD(ON) ─ 5V

0,		if vi < VD(ON) ─ 5V

Consequently, the output will be given by,

{

V0 =

Let us sketch the output of the series clipper circuit above for a sinusoidal input, using the

ideal diode model, as shown below.

Applications of Clippers

- Clippers are used for the generation and shaping of waveforms.

- Clippers are used for the protection of circuits from spikes.

- Clippers are used for amplitude restorers.

- Clippers are used as voltage limiters.

- Clippers are used in television circuits.

- Clippers are used in FM transmitters.

Clamper

Clamper circuits are used to clamp an input signal to a different dc level. It basically adds dc component to the applied input signal in order to push the signal to either the positive or negative side. Clamper circuit is a combination of a resistor along with a diode and capacitor. It sometimes also employs dc battery so as to have an additional shift in the signal level. Clampers are also known as DC voltage restorers or level shifter.

Clamper circuits are constructed in a similar manner as that of clipper circuits. However, clamper includes an extra charging element that is the capacitor in its circuitry. The combination of resistor and capacitor in the clamper circuit is used to maintain different DC level at the output of the clamper.

The working of clamper circuits depends on the variation in the time constant of the capacitor. This variation is the outcome of changing the current path of the diode with the change in input signal polarity.

Types of clampers:

Clampers are further divided into only two types:

1. Positive Clamper

2. Negative Clamper

1. Positive Clamper: In positive clamper circuit, the peak of the signal is raised above to the zero level.

Fig 2.5: Positive Clamper

2. Negative Clamper: In negative clamper circuit, the peak of the signal is shifted below the zero level.

Fig 2.5: Negative Clamper

NOTE: The clamper cannot be dual or combination. The waveform can be shifted to a certain limit by introducing a factor called biasing. Here, the cells are added to the circuit parallel to the input signal for the purpose of biasing. This makes sure only the desired portion is shifted either above or below the zero level. This further classifies the positive and negative clampers into following parts:

- Biased Positive Clamper

- Biased Negative Clamper

The working of the biasing can be understood with some of the following solved problems and examples.

How to solve clipper circuits?

As we know. clamper circuits clamp a signal to different DC levels. The circuit must have a capacitor, a diode, and a resistive element as shown below, but it can also employ an independent DC supply to introduce an additional shift.

Throughout the analysis, we will assume that for all practical purposes the capacitor will fully discharge in five time constants, i.e., 5Tdischarge, where time constant is Tdischarge = RC.

The magnitude of R and C must be chosen such that the time constant Tdischarge = RC is large enough to ensure that the voltage across the capacitor does not discharge significantly during the interval the diode is nonconducting.

Diode state=

{

ON;	if vi  ≥ VD(ON) + Vc

OFF;	if vi < VD(ON) + Vc

For the circuit above, the diode state is given by

where VC is the voltage across the capacitor C.

The clamper output is given by

V0 = vi ─ VC

Consequently, when the capacitor is charged at the maximum voltage, i.e.,

VC = Vm ─ VD(ON) and diode is OFF, the clamper output will be

vo = vi ─ ( Vm ─ VD(ON) )

IMPORTANT: The clamper shifts the signal in the direction of the diode arrowhead by an amount of ( Vm ─  VD(ON) )

Let us generate steps for the clamper operation for the circuit above, assuming capacitor is fully discharged initially, i.e., at t = 0, using the ideal diode model.

1. The diode will be ON in the first positive half cycle as VC = 0 and the capacitor will charge up very quickly to the peak value Vm with a time constant

Tcharge = Crav ≅ 0 where rav ≅ 0 Ω is the internal resistance of the diode as shown below.

2. Once the capacitor is charged, i.e., VC = Vm. Then the diode turns OFF, as shown below.

3. Capacitor will discharge until the input waveform gets larger than the capacitor voltage, i.e., vi(t) > vC (t). Thus, the discharging period ( T2 ) must be much smaller than the fully discharge constant 5Tdischarge in order to keep the voltage across the capacitor almost constant at VC ≅ Vm, i.e.,

5Tdischarge ≫ T2

So, if take ( 5Tdischarge ≥  50 T2 ), then we obtain the following condition for the clamping operation

Tdischarge ≥ T2

where T is the period of the input signal vi.

For a clamping operation, selected capacitor C and resistor R values should satisfy

the discharge condition above.

IMPORTANT: If it is not explicitly stated we are going to assume that capacitor is already charged, e.g., VC = Vm─ VD(ON)

An Example:

Consider the clamping circuit below and plot the output waveform. Assume the diode is ideal.

Solution: As this is a clamping circuit, i.e., Tdischarge ≥ 5T, we obtain the following output

vo = vi ─ V + VD(ON)

where VD(ON) = 0 V. The resulting waveform is shown below.

Applications of Clampers

- Clampers are used as direct current restorers.

- Clampers are used to remove distortions.

- Clampers are used as voltage multipliers.

- Clampers are used for the protection of amplifiers.

- Clampers are used as test equipment.

- Clampers are used as base-line stabiliser

References

1. Introduction- [https://en.wikipedia.org/wiki/Diode]

2. Clipper-[https://electronicscoach.com/clipper-circuits.html]

3. Clipper types [https://www.tutorialspoint.com/electronic_circuits/electronic_positive_clipper_circuits.htm]

3. clapper- [https://electronicscoach.com/clamper-circuits.html]

4.clappertypes-[https://www.tutorialspoint.com/electronic_circuits/electronic_clamper_circuits.htm]

5. Applications-

[https://www.elprocus.com/types-of-clipper-and-clamper-circuits-and-applications/]

[https://www.electronicshub.org/diode-clippers-and-clampers/]