A Mini Project Report on

ENGINEERING MECHANICS

Submitted in partial fulfilment of the requirement for the Degree of

B. Tech.

in

Computer Science and Technology

Submitted By:

Members of Group 22, CSE ‘K’

Prottay Kumar Adhikary	: 1912157

Saurabh Sinha	: 1912159

Subhojit Ghimire	: 1912160

Under the guidance of:

Dr. Simanchal Kar

Assistant Professor

Department of Mechanical Engineering

NIT Silchar

10th July, 2020

ME 101

Engineering Mechanics

1912157, 1912159, 1912160

CSE

2nd

July 2020

Semester_______ Branch__________Regn. No_______________________

Subject_____________________________ Subject Code_______________

Date of Submission _____________________________________________

END SEMESTER MINI PROJECT ______________________________________

NATIONAL INSTITUTE OF TECHNOLOGY, SILCHAR

Group I

1.

i) Ans) [Plane Truss] plane truss

ii) Ans) [Kinetics of Particles] Kinetics

iii) Ans) [Wedge Friction] perpendicular

iv) Ans) [Theorem of Pappus-Guldin] surface area and the volume of the revolving body.

v) Ans) [Statics of Rigid Body] constant

vi) Ans) [Kinetics of Particles] Rectilinear motion

vii) Ans) [Principle of Work and Energy] conservative force

viii) Ans) [Principle of Impulse and Momentum] inertia and angular speed

ix) Ans) [Static Equilibrium] Lami’s theorem

x) Ans) [Varignon’s Theorem] Varignon’s theorem

2.

i) Ans) [Simple Screw Jack] True.

ii) Ans) [Belt Friction] False. The frictional force increases.

iii) Ans) [Beams- Types of Loads] False. It is the cantilever beam. A simply supported beam is the one which carries two reaction forces at its two ends and a point load at its mid-point.

iv) Ans) [Couple] False. Couple is a pair of equal parallel forces that are opposite in direction.

v) Ans) [Moment of Inertia] True.

vi) Ans) [Direct and Oblique Collision] True.

vii) Ans) [Principle of Work and Energy] False. The work done is equal to the change in kinetic energy.

viii) Ans) [Kinetics of Rotation Motion] True.

ix) Ans) [Moment of Inertia] False. Parallel axis for any area is used to add two mutually perpendicular moment of inertias for areas.

x) Ans) [Parallel and Perpendicular Axes Theorem] True.

Group II

3.

i) Ans) [Statics of Rigid Body] Option c.

Solution:

Fx=0 : -T1sin30°+ T2cos30°=0		- (i)

Fy=0 : T1cos30°- T2sin30°=2500	- (ii)

On solving equations (i) and (ii), we get,

T1 = 4330N

And, 	T2 = 2500N

Fx=0 :    -T2sin60°+ T3sin60° = 0

T3sin60°= T2sin60°

∴ T3= T2=2500N

Therefore, T1=4330N, T2=T3=2500N

ii) Ans) [Moment of Force] Option b.

Solution:

Fx=0 :            RB-XA=0

∴RB=XA		- (i)

Fy=0 :            YA-200=0

∴YA=200N		- (ii)

MA=0 : RB0.04-200 (0.2)=0

∴ RB=1000N

∴ XA=1000N	(from i)

RA=XA2+YA2=10002+2002=1019.8N≈1020N

Therefore, Forces exerted on cylinder at points A and B are 1020N and 1000N, respectively.

iii) Ans) [Lami’s Theorem] Option c.

Solution:

Using Lami’s theorem,

200sinα=70sin150°=Fsin90°

So, F=70 sin90°sin150°=700.5=140N

And, 360°-90°+150°= α

∴ α=120°

Therefore, F=140N and α=120⁰.

iv) Ans) [Belt Friction] Option b.

Solution:

For the belt drive,

F=μRN=T+δTcosδθ2-Tcosδθ2

For small angle δθ, cosδθ2=1, so,

F=μRN=T+δT-T=δT

∴RN=δTμ

Also, we have, RN=Tδθ

Equating the values of RN, we get,

δTT=μδθ

Integrating between the limits T2 and T1 and from 0 to θ,

we have,

T1T2δTT=μ0θδθ

∴logeT1T2= μθ

∴T1T2= eμθ is the required ratio

v) Ans) [Plane Truss] Option b.

Solution:

Deficient truss is the truss consisting of 8 joints, 10 members and 4 reaction components.

vi) Ans) [Kinematics of Particles] Option d.

Solution:

Given,		Vrg=2 km/h

Vmr=3 km/h

We have,	Vmg=Vmr+ Vrg

0=-3sinθ+2

∴sinθ=23

∴cosθ=53

So, 		t=xv=0.53cosθ=0.53* 53=125 h

vii) Ans) [Projectile Motion] Option a.

Solution:

Given, 		v = 20 m/s

α = 40⁰

Minimum velocity in a projectile motion is obtained at the top.

Vmin = u cos α = 20 cos40⁰ = 15.32 m/s

viii) Ans) [Kinetics of Rotational Motion] Option b.

Solution:

The acceleration of a particle at any instant moving along a circular path in a direction normal to the tangent at that instant and directed towards the center of the circular path is known as normal component of the acceleration or normal acceleration. It is also called radial or centripetal acceleration.

ix) Ans) [Kinematics of Plane Motion] Option b.

Solution:

We know,

s=dt

Here, d = 60 miles

And,   t = (4 pm – 2 pm = 2 hours)

Therefore,

s= 602=30 m/h

x) Ans) [Principle of Impulse and Momentum] Option b.

Solution:

We know,	momentum = mass x velocity.

Given, both the objects have same momentum, i.e., momentum is constant.

Hence, mass ∝ 1velocity

i.e., lighter the object, more the velocity. So, the lighter object will have greater velocity

Group III

4.

i) Solution) [Plane Truss- Method of Sections]

MA=0 : RB6-392=0

∴RB=10KN

In ΔCGD,

tanθ=DGCG=12 => θ=26.56°

To find axial force FCD,

RB2-FCDcosθ3=0

FCD=-10*2cos26.56°*3=-7.45KN

∴FCD=7.45KN (compressive)

To find axial forces FCF and FEF,

Fx=0: -FEF-FCFcos45°-FCDcos26.56°=0

FEF+FCFcos45°=7.45cos26.56°=6.67	-(i)

Fy=0: RB+FCFsin45°-FCDsin26.56°=0

FCF=FCDsin26.56°-10sin45°=-13.33sin45°=-18.85KN

∴FCF=18.85KN (compressive)

From equation (i),

FEF=6.67+18.85cos45°=20KN

∴FEF=20KN (Tensile)

Therefore, Forces acting in members CD, EF and CF are 7.45KN (C), 20KN (T) and 18.85KN, respectively.

ii) Solution) [Plane Truss- Method of Joints]

∑MA = 0,

Rc (3lcos30°) – W (l) =0

Or, Rc (3×2√3) =W

∴ Rc =W2√3

In ∆ABC,

Cos 30°= 3lAC

Or, AC= 3lcos30°

Joint C

∑Fy = 0,

–FGC –FBC cos 60° = 0    (i)

∑Fy = 0,

Rc + FBC sin 60° = 0

Or, FBC = -Rcsin60°

Or, FBC = -W2√3 × 1√32

Or, FBC = -W3

∴ FBC = W3 (Compressive)

Equation becomes,

FGC= W3 × 12

∴ FGC= W6 (Tensile)

In ∆AFG,

tan30° = FG2l

or, 1√3 = FG2l

∴ FG = 2l√3

In ∆BFG,

tanθ = FGl

or, tanθ = 1l × 2l√3

or, tan = 2√3

θ = tan-1(2√3)

∴ θ = 49.1°

Joint B:

∑Fy = 0,

–FBF – FBG cosθ = 0    (ii)

∑Fy = 0,

–FBC – FBG sinθ = 0

Or, -W3 = FBG sinθ

Or, FBG = W3sinθ

∴ FBG = 0.44 W (tensile)

Equation (ii) becomes,

–FBF = FBG cosθ

Or, –FBF = 0.44 W × cos(49.1°)

Or, FBF = -(0.288)W

∴ FBF = 0.288 W (compressive)

Joint G:

∑Fy = 0,

FGC–FEG – FBG cos(79.1°) – FGF cos(60°)= 0

Or, W6 + 0.44 W×cos(79.1°)= FEG + FGF cos(60°)

∴ FEG + FGF cos(60°) = 0.2498 W             (iii)

∑Fy = 0,

FBG sin(79.1°) + FGF sin(60°)= 0

Or, 0.44 W ×sin(79.1°) sin(60°) = – FGF

Or, FGF = – 0.4989 W

Or, FGF = – 0.5 W

∴ FBG = 0.5 W (Compressive)

Equitation (iii) becomes,

FEG = FBG cosθ

Or, FEG = 0.2498 W + 0.5 × cos(60°)

∴ FEG = 0.5 W (compressive)

iii) Solution) [Virtual Work]

We know,

Total weight 4W acts on O.

Let, T be the thrust in the rod EF. Joining midpoint of AB and AD.

Here,

Point of suspension A is fixed.

Let, length of rod is 2a.

So, ∠ BAO= α

In ∆ABO,

AOAB = cosα

Or, AO2a = cosα

∴ AO= 2a cosα   (i)

Now for EF,

As EF = 2EK

In ∆AEK,

EF= 2EK

∵ EKEA = sinα

Or, EK= sinα

∴ EF= 2asinα = l (say)

Now, give the system a small virtual displacement such that α becomes α+𝛿α and l becomes l+𝛿l. Then the equation of the virtual work is-

4W. (AO) + T. 𝛿l = 0

Or, ∆W (2a cosα) + T𝛿 (2a sinα)= 0

Or, 8Wa sinα 𝛿 α + T 2a cosα. 𝛿α= 0

∴ T= 4W. tanα   [∵𝛿α≠0]

iv) Solution) [Friction]

Given, µA=0.3, µB=0.2

From FBD of Block A,

Fx=0:  μANA-R-WAsinθ=0

μANA-WAsinθ=R 	- (i)

FY=0:  NA-WAcosθ=0

NA=WAcosθ	- (ii)

From equations (i) and (ii), and putting known values, we get,

0.3250cosθ-250sinθ=R

75 cosθ – 250 sinθ = R	- (iii)

From FBD of Block B,

Fx=0:  μBNB+R-150sinθ=0

150sinθ-μBNB=R		- (iv)

Fx=0:  NB-150cosθ=0

NB=150cosθ		- (v)

From equations (iv) and (v), and putting known values, we get,

150sinθ-0.2150cosθ=R

R=150sinθ-30cosθ			- (vi)

From equations (iii) and (vi),

150sinθ-30sinθ=75cosθ-250sinθ

400sinθ=105cosθ

tanθ=105400

∴θ=14.7°

Therefore, 14.7⁰ is the maximum angle that can be reached before bodies slip down the incline.

v) Solution) [Ladder Friction]

Given, μB=13=0.33, μA=12=0.5

Fx=0:     NB- μBNB=0

NB=0.5NA=12NA	- (i)

Fy=0:      NA+ μBNB-W-W2=0

NA+1312NA=3W2

76NA=32W

∴NA=97W

From (i),

NB=1297W=914W			- (ii)

MA=0:     -NBOB-μBNBOA+WAG+W2(AM1)=0	- (iii)

In ΔAOB, AB=20m,

sin45°=OBAB =>OB=ABsin45°=14.14m

cos45°=OAAB =>OA=ABcos45°=14.14m

In ΔAC1C,

cos45°=AC1AC =>AC1=ACcos45°=7.07m

In ΔAM1M,

cos45°=AM1AM =>AM1=xcos45°

From equation (ii) and (iii), putting values we obtained, we get,

-914W14.14-13914W14.14+W7.07+W2 xcos45°=0

-9.09-3.03+7.07+0.35x=0

0.35x=5.05

∴x=14.43m

vi) Solution) [Projectile Motion]

Known Values,	yo = 100m;		vo = 30m/s;

θ = 20⁰;		xo = 0m

(a) To find range,

At impact, y=0m

We know,

y=12gt2+vosinθt+yo

0=12gt2+vosinθt+yo
			Using the quadratic formula,

t=-vosinθ±(vosinθ)2-2gyog

This gives,

either, t = -3.59 	(time cannot be in negative, so no valid)

or, t = 5.68 s

Again,

x=vocosθt=30cos20°5.68

∴x=160.1m

Therefore, the horizontal range covered by the projectile is 160.1m.

(b) To find peak height,

At peak height, vg = 0m/s

We know,

vg=vosinθ+gt

0=vosinθ+gt

t=-vosinθg=-30sin20°-9.8=1.05

Also,

ymax=12gt2+vosinθt+yo

ymax=12-9.81.052+30sin20°1.05+100

∴ymax=105.4m

Therefore, the peak height attained by the projectile is 105.4m.

vii) Solution) [Direct and Oblique Collision]

Firstly, we know,

m1v1+m2v2=m1v1'+m2v2'

4*5+0=4v1'+2v2'

4v1'+2v2'=5			- (i)

Also,

v1+v1'=v2+v2'

5+v1'=0+v2'

-v1'+v2'=5				- (ii)

Multiplying (ii) by 4 and then adding (i), we get,

6v2'=40

∴v2'=6.67 m/s

∴v1'=1.67 m/s

Checking Condition I by putting the obtained values for v1' and v2',

m1v1+m2v2=m1v1'+m2v2'

45+0=41.67+2(6.67)

∴20≈20.02

Checking Condition II, by putting the obtained values for v1' and v2',

v1+v1'=v2+v2'

5+1.67=0+6.67

∴6.67=6.67

Checking Condition III, by putting the obtained values for v1' and v2',

12m1v12+12m2v22=12m1v1'2+12m2v2'2

12(4)52+12(2)02=12(4)1.672+12(2)6.672

50+0=5.55778+44.4889

50≈50.0667

viii) Solution) [Projectile Motion]

We have,

x=ucosθt					- (i)

y=usinθt-12gt2				- (ii)

Replacing value of t from (i) in (ii),

y=(usinθ)(xucosθ)-12g(xucosθ)2

y=xtanθ-12gx2u2cos2θ			- (iii)

(a) To find time of flight:

For this, putting y = 0 in equation (ii), we get,

Time of flight, t=2usinθg

(b) To find the maximum vertical distance (height) covered,

v2=u2-2gH

0=(usinθ)2-2gH

∴Maximum height reached,  H=u2sin2θ2g

(c) To find the maximum horizontal distance covered,

R=(ucosθ)×t=ucosθ×2usinθg

∴Maximum horizontal distance travelled,  R=u2sin2θg

ix) Solution) [Kinetics of Particles]

We know,

Fg = mg sinθ

So,	Fg = 8 (9.8) sin 30⁰ = 39.2N

(a)	Now,

a=FM=m2g-m1gsinθm1+m2

So,	a=5 9.8-89.8sin30°13=49-39.213=9.813=0.754m/s2

(b)	Also,

Fy=T-m2g

-m2a=T-m2g

T=m2g-m2a=5 9.8-5 0.754=45.23N

And,

Fx=T-Fg

m1a=T-m1gsinθ

T=m1a+m1gsinθ=8 0.754+8 9.8sin30°=45.23N

Therefore, the tension in the rope is 45.23N.

(c)

To find a hanging mass that will balance the system,

We have,

m2g=Fg

m2g=m1gsinθ

m2=m1sinθ

m2=8sin30°=8 12=4 kg

Therefore, a hanging mass of 4 kg will cause the system to remain at rest.

x) Solution) [Kinematics of Plane Motion]

Firstly for car P,

Known values,

u = 16.67 m/s

a  = -2.5 m/s2 (for deceleration)

Let, s = x.

Using,

s=ut+12at2

x=16.67 t+12(-2.5)t2		- (i)

Now, for car Q,

Known values,

u = 16.67 m/s

s = x + 250

a = 2 m/s2

Using,		s=ut+12at2

x+250=16.67 t+122t2

x=16.67 t+t2-250		- (ii)

Equating (i) and (ii), we get,

t = 10.541 s

Substituting the value of t in (i), we get,

x = 36.827 m

Therefore, The car Q will overtake car P at the distance of 36.827m from the instant, which is equivalent to 10.541 seconds.